Products:

2. Lined Fittings

3. Lined  Instrument Tee

VescoaT supplies and manufacturer of FEP / PFA Lined Instrument Tee of various Standard, Non Standard size on Request, We also manufactures in PFA / FEP / HDPE lined Instrument Tee as per international standard of ASTM F1545.

| Dimension Detail: |  |  | All Dimensions are in mm |  |  |
| --- | --- | --- | --- | --- | --- |
| NB 1 |  | NB 2 |  | H | L |
| MM | INCH | MM | INCH |  |  |
| 25 | 1" | 25 | 1" | 89 | 50 |
| 40 | 1.5" | 25 | 1" | 102 | 50 |
|  |  | 40 | 1.5" | 102 | 65 |
| 50 | 2" | 25 | 1" | 114 | 50 |
|  |  | 40 | 1.5" | 114 | 65 |
|  |  | 50 | 2" | 114 | 77 |
| 65 | 2.5" | 25 | 1" | 127 | 50 |
|  |  | 40 | 1.5" | 127 | 65 |
|  |  | 50 | 2" | 127 | 77 |
| 80 | 3" | 25 | 1" | 140 | 50 |
|  |  | 40 | 1.5" | 140 | 65 |
|  |  | 50 | 2" | 140 | 77 |
| 100 | 4" | 25 | 1" | 165 | 50 |
|  |  | 40 | 1.5" | 165 | 65 |
|  |  | 50 | 2" | 165 | 77 |
| 150 | 6" | 25 | 1" | 203 | 50 |
|  |  | 40 | 1.5" | 203 | 65 |
|  |  | 50 | 2" | 203 | 77 |
| 200 | 8" | 25 | 1" | 229 | 50 |
|  |  | 40 | 1.5" | 229 | 65 |
|  |  | 50 | 2" | 229 | 77 |
| 250 | 10" | 25 | 1" | 279 | 50 |
|  |  | 40 | 1.5" | 279 | 65 |
|  |  | 50 | 2" | 279 | 77 |
| 300 | 12" | 25 | 1" | 305 | 50 |
|  |  | 40 | 1.5" | 305 | 65 |
|  |  | 50 | 2" | 305 | 77 |

Technical Specification:

| Size | 15NB TO 300NB ( ¾” TO 12” ) |
| --- | --- |
| Length | As per Standard of B16.5 & ANSI B 16.42 |
| Liner | PFA - Per Fluoro Alkoxy, ASTM D3307 FEP - Fluorinated Ethylene Propylene, ASTM D2116 HDPE - As per Standard |
| Body | As per ASTM A 234 Gr. WPB / DI A395 / SS304 / SS316 ( Seamless) |
| Steel Housing | Carbon steel or Stainless Steel |
| Flanges | ASTM A 105 (N) / IS 2062 / ASTM A351 GR CF8 & CF8M |
| Drilling / Pressure | ASA 150# / 300# / DIN / Table – F |
| Dimensional STD | Tee : B16.5 & ANSI B 16.42 Flanges : ASME B 16.5 Lining Standard : ASTM F 1545 |
| Temperature | -29℃~200℃ |
| Vacuum | Full vacuum rated |

| Testing/Inspection |  | Mechanical Properties |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | PTFE | PFA | FEP |
| Hydro Test | At 29 Bar, after lining | Tensile Strength, PSI (min) | 3500 | 3800 | 3000 |
| Spark Test | At 25 KV | Elongation, % (min) | 300 | 300 | 250 |