Products:

2. Lined Fittings

6. Lined Reducer ( Concentric / Eccentric )

VescoaT supplies and manufacturer of FEP / PFA Lined Reducer of various Standard, Non Standard size on Request, We also manufactures in PFA / FEP / HDPE lined Reducer in Both Concentric and Eccentric Based on Application and international standard of ASTM F1545.

| Dimension Detail: |  | All Dimensions are in mm |  |  |
| --- | --- | --- | --- | --- |
| NB 1 |  | NB 2 |  | L |
| MM | INCH | MM | INCH |  |
| 40 | 1.5" | 25 | 1" | 114 |
| 50 | 2" | 25 | 1" | 127 |
|  |  | 40 | 1.5" |  |
| 65 | 2.5" | 25 | 1" | 127 |
|  |  | 40 | 1.5" |  |
|  |  | 50 | 2" |  |
| 80 | 3" | 25 | 1" | 152 |
|  |  | 40 | 1.5" |  |
|  |  | 50 | 2" |  |
|  |  | 65 | 2.5" |  |
| 100 | 4" | 25 | 1" | 178 |
|  |  | 40 | 1.5" |  |
|  |  | 50 | 2" |  |
|  |  | 65 | 2.5" |  |
|  |  | 80 | 3" |  |
| 150 | 6" | 25 | 1" | 229 |
|  |  | 40 | 1.5" |  |
|  |  | 50 | 2" |  |
|  |  | 65 | 2.5" |  |
|  |  | 80 | 3" |  |
|  |  | 100 | 4" |  |
| 200 | 8" | 25 | 1" | 279 |
|  |  | 40 | 1.5" |  |
|  |  | 50 | 2" |  |
|  |  | 65 | 2.5" |  |
|  |  | 80 | 3" |  |
|  |  | 100 | 4" |  |
|  |  | 150 | 6" |  |
| 250 | 10" | 25 | 1" | 305 |
|  |  | 40 | 1.5" |  |
|  |  | 50 | 2" |  |
|  |  | 65 | 2.5" |  |
|  |  | 80 | 3" |  |
|  |  | 100 | 4" |  |
|  |  | 150 | 6" |  |
|  |  | 200 | 8" |  |
| 300 | 12" | 25 | 1" | 356 |
|  |  | 40 | 1.5" |  |
|  |  | 50 | 2" |  |
|  |  | 65 | 2.5" |  |
|  |  | 80 | 3" |  |
|  |  | 100 | 4" |  |
|  |  | 150 | 6" |  |
|  |  | 200 | 8" |  |
|  |  | 250 | 10" |  |

| 150# Class |  |  | All Dimensions are in mm |  |  |
| --- | --- | --- | --- | --- | --- |
| NB |  | d4 | K | D | No. of Holes & Hole Dia |
| MM | INCH |  |  |  |  |
| 25 | 1" | 50.8 | 79.2 | 108 | 4 x 15.9 |
| 40 | 1.5" | 73.2 | 98.6 | 127 | 4 x 15.9 |
| 50 | 2" | 91.9 | 120.7 | 152.4 | 4 x 19.0 |
| 65 | 2.5" | 104.6 | 139.7 | 177.8 | 4 x 19.0 |
| 80 | 3" | 127.0 | 152.4 | 190.5 | 4 x 19.0 |
| 100 | 4" | 157.2 | 190.5 | 228.6 | 8 x 19.0 |
| 150 | 6" | 215.9 | 241.3 | 279.5 | 8 x 22.2 |
| 200 | 8" | 269.7 | 298.5 | 342.9 | 8 x 22.2 |
| 250 | 10" | 323.9 | 362.0 | 406.4 | 12 x 25.4 |
| 300 | 12" | 381.0 | 431.8 | 482.6 | 12 x 25.4 |

Technical Specification:

| Size | 15NB TO 300NB ( ¾” TO 12” ) |
| --- | --- |
| Length | As per Standard of B16.5 & ANSI B 16.42 |
| Liner | PFA - Per Fluoro Alkoxy, ASTM D3307 FEP - Fluorinated Ethylene Propylene, ASTM D2116 HDPE - As per Standard |
| Body | As per ASTM A 234 Gr. WPB / DI A395 / SS304 / SS316 ( Seamless) |
| Steel Housing | Carbon steel or Stainless Steel |
| Flanges | ASTM A 105 (N) / IS 2062 / ASTM A351 GR CF8 & CF8M |
| Drilling / Pressure | ASA 150# / 300# / DIN / Table – F |
| Dimensional STD | Reducer : B16.5 & ANSI B 16.42 Flanges : ASME B 16.5 Lining Standard : ASTM F 1545 |
| Temperature | -29℃~200℃ |
| Vacuum | Full vacuum rated |

| Testing/Inspection |  | Mechanical Properties |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | PTFE | PFA | FEP |
| Hydro Test | At 29 Bar, after lining | Tensile Strength, PSI (min) | 3500 | 3800 | 3000 |
| Spark Test | At 25 KV | Elongation, % (min) | 300 | 300 | 250 |

Notes :

1. All Reducer have SOFF Fix flange on all size

( one end loose flange and fixed flange at the other end. Or Both Side Loose Flange as on Request)

2. Suitable vent holes are provided for all fittings.

3. All Fittings work will be supplied with suitable wooden end covers to protect the flare faces.

4. Material test certificates will be provided for all raw materials

5. The plastic liner flare will replace the raised face portion of the flange.