Products:

2. Lined Fittings

8. Lined Blind Flange

VescoaT supplies and manufacturer of PTFE / FEP / PFA Lined Blind Flange of various sizes as per international standard of ASTM F1545.

| Dimension Detail: 150# Class |  |  |  | All Dimensions are in mm |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NB |  | d4 | K | D | H | No. of Holes & Hole Dia |
| MM | INCH |  |  |  |  |  |
| 15 | 1/2" | 35.1 | 60.5 | 88.9 | 11.1 | 4 x 15.9 |
| 20 | 3/4" | 42.9 | 69.9 | 94.4 | 12.5 | 4 x 15.9 |
| 25 | 1" | 50.8 | 79.2 | 108 | 14.5 | 4 x 15.9 |
| 40 | 1.5" | 73.2 | 98.6 | 127 | 17.5 | 4 x 15.9 |
| 50 | 2" | 91.9 | 120.7 | 152.4 | 19.5 | 4 x 19.0 |
| 65 | 2.5" | 104.6 | 139.7 | 177.8 | 22.2 | 4 x 19.0 |
| 80 | 3" | 127.0 | 152.4 | 190.5 | 24.0 | 4 x 19.0 |
| 100 | 4" | 157.2 | 190.5 | 228.6 | 24.0 | 8 x 19.0 |
| 150 | 6" | 215.9 | 241.3 | 279.5 | 25.5 | 8 x 22.2 |
| 200 | 8" | 269.7 | 298.5 | 342.9 | 29.0 | 8 x 22.2 |
| 250 | 10" | 323.9 | 362.0 | 406.4 | 30.5 | 12 x 25.4 |
| 300 | 12" | 381.0 | 431.8 | 482.6 | 32.0 | 12 x 25.4 |

Technical Specification:

| Size | 15NB TO 300NB ( ¾” TO 12” ) |
| --- | --- |
| Length | As per Standard of B16.5 & ANSI B 16.42 |
| Liner | PFA - Per Fluoro Alkoxy, ASTM D3307 FEP - Fluorinated Ethylene Propylene, ASTM D2116 HDPE - As per Standard |
| Body | As per ASTM A 234 Gr. WPB / DI A395 / SS304 / SS316 ( Seamless) |
| Steel Housing | Carbon steel or Stainless Steel |
| Flanges | ASTM A 105 (N) / IS 2062 / ASTM A351 GR CF8 & CF8M |
| Drilling / Pressure | ASA 150# / 300# / DIN / Table – F |
| Dimensional STD | Flanges : ASME B 16.5 Lining Standard : ASTM F 1545 |
| Temperature | -29℃~200℃ |
| Vacuum | Full vacuum rated |

| Testing/Inspection |  | Mechanical Properties |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | PTFE | PFA | FEP |
| Hydro Test | At 29 Bar, after lining | Tensile Strength, PSI (min) | 3500 | 3800 | 3000 |
| Spark Test | At 25 KV | Elongation, % (min) | 300 | 300 | 250 |

Notes :

1. All Fittings work will be supplied with suitable wooden end covers to protect the flare faces.

2. Material test certificates will be provided for all raw materials

3. The plastic liner flare will replace the raised face portion of the flange.